### Arduino Library for Processing (and Firmata)

This library allows you to control an Arduino board from Processing without writing code for the Arduino. Instead, you upload a standard firmware (program) to the board and communicate with it using the library. The firmware is called Firmata, and is included in the Arduino software. The corresponding Processing library can be downloaded below.

#### Download

Library for Processing v2.0: processing2-arduino.zip (Updated 6 Nov. 2013) 
(properties file here: processing2-arduino.txt)

Library for Processing v1.5: processing-arduino.zip (Updated 11 Nov. 2011) 
(properties file here: processing-arduino.txt)

Note: if you run Linux, you need to change Arduino.jar into arduino.jar, because Linux is case sensitive and it does not work if you don't change this letter (Arduino.jar is in the folder "library" of this Processing Library).

#### Instructions

- Unzip the library and copy the "arduino" folder into the "libraries" sub-folder of your Processing Sketchbook. (You can find the location of your Sketchbook by opening the Processing Preferences. If you haven't made a "libraries" sub-folder, create one.)

- Run Arduino, open the Examples > Firmata > StandardFirmata sketch, and upload it to the Arduino board.

- Configure Processing for serial:http://processing.org/reference/libraries/serial/

- In Processing, open one of the examples that comes with with the Arduino library.

- Edit the example code to select the serial port used by Arduino. Specifically, change the [0] in this line

- arduino = new Arduino(this, Arduino.list()[0], 57600);

To find the correct item in the array, run this code in Processing:

import processing.serial.*;

import cc.arduino.*;

println(Arduino.list());

The output window will enumerate your serial ports. Select the number corresponding to the serial port in your Arduino environment found under Tools > Serial Port.

- Run the example.

#### Reference

These functions are in the Processing Arduino Library and communicate (from Processing) with a Arduino, upon which the Firmata sketch has been installed.

Arduino.list(): returns a list of the available serial devices. If your Arduino board is connected to the computer when you call this function, its device will be in the list.

Arduino(parent, name, rate): create an Arduino object. Parent should be "this" (without the quotes); name is the name of the serial device (i.e. one of the names returned by Arduino.list()); rate is the speed of the connection (typically 57600). Note that in the v2 library, the rate parameter is optional.

pinMode(pin, mode): set a digital pin to input, output, or servo mode (Arduino.INPUT, Arduino.OUTPUT, or Arduino.SERVO).

digitalRead(pin): returns the value of a digital pin, either Arduino.LOW or Arduino.HIGH (the pin must be set as an input).

digitalWrite(pin, value): writes Arduino.LOW or Arduino.HIGH to a digital pin.

analogRead(pin): returns the value of an analog input (from 0 to 1023).

analogWrite(pin, value): writes an analog value (PWM wave) to a digital pin that supports it (pins 3, 5, 6, 9, 10, and 11); value should be from 0 (always off) to 255 (always on).

servoWrite(pin, value): writes a value to a servo motor; value should be from 0 to 180.

#### Example

import processing.serial.*;

import cc.arduino.*;

Arduino arduino;

int ledPin = 13;

void setup()

{

//println(Arduino.list());

arduino = new Arduino(this, Arduino.list()[0], 57600);

arduino.pinMode(ledPin, Arduino.OUTPUT);

}

void draw()

{

arduino.digitalWrite(ledPin, Arduino.HIGH);

delay(1000);

arduino.digitalWrite(ledPin, Arduino.LOW);

delay(1000);

}

#### Another example

- Hacking a Processing Sketch for Firmata (Firmata 101)

#### Troubleshooting

If you're having trouble getting this to work, you can ask for help in the Arduino Forum. If you've found a bug in the Arduino (Firmata) library for Processing, please report it on the GitHub issues list.

### External Resources

- Arduino meets processing from K3, malmoe with several examples for creating graphics from digital and analog sensor input

- Tom Igoe's section on Processing with several communication and signal postprocessing examples

- Pp. 169-176 in the "Physical Computing" book, by Dan O'Sullivan and Tom Igoe (preview in Google books)

- You can also use the SimpleMessageSystem to communicate with Processing. Check the example in the download.

Manuals and Curriculum

Installing Arduino on Linux

Board Setup and Configuration

Development Tools

Arduino on other Atmel Chips

Interfacing With Hardware

- Output

- Input

- User Interface

- Storage

- Communication

- Power supplies

- General

Interfacing with Software

User Code Library

- Snippets and Sketches

- Libraries

- Tutorials

Suggestions & Bugs

Electronics Technique

Sources for Electronic Parts

Related Hardware and Initiatives

Arduino People/Groups & Sites

Exhibition

Project Ideas

Languages

PARTICIPATE

- Suggestions

- Formatting guidelines

- All recent changes

- PmWiki

- WikiSandBox training

- Basic Editing

- Cookbook (addons)

- Documentation index

- Drone with Arduino

- Thermostat with Arduino

- Arduino On StackExchange

## Hacking a Processing Sketch for Firmata (Firmata 101)

by Paul Badger

What you should have done so far.

- Unpacked and installed Firmata on your Freeduino*

- Quit the Arduino IDE

- Unpacked and installed the Arduino library in Processing/libraries.

Open the Processing Sketch "Examples -> Topics -> Fractals and L-Systems -> Tree

Run the code and try out the sketch - Very groovy, no? Look over the code and see where the mouse input gets into the sketch.

float a = (mouseX / (float) width) * 90f;

This might be a good place to intervene to hook the sketch up to outside input.

Here's the code for the whole sketch

/**

* Recursive Tree

* by Daniel Shiffman.

*

* Renders a simple tree-like structure via recursion

* Branching angle calculated as a function of horizontal mouse location

*/

float theta;

void setup() {

size(200,200);

smooth();

}

void draw() {

background(0);

frameRate(30);

stroke(255);

// Let's pick an angle 0 to 90 degrees based on the mouse position

float a = (mouseX / (float) width) * 90f;

// Convert it to radians

theta = radians(a);

// Start the tree from the bottom of the screen

translate(width/2,height);

// Draw a line 60 pixels

line(0,0,0,-60);

// Move to the end of that line

translate(0,-60);

// Start the recursive branching!

branch(60);

}

void branch(float h) {

// Each branch will be 2/3rds the size of the previous one

h *= 0.66f;

// All recursive functions must have an exit condition!!!!

// Here, ours is when the length of the branch is 2 pixels or less

if (h > 2) {

pushMatrix();    // Save the current state of transformation (i.e. where are we now)

rotate(theta);   // Rotate by theta

line(0,0,0,-h);  // Draw the branch

translate(0,-h); // Move to the end of the branch

branch(h);       // Ok, now call myself to draw two new branches!!

popMatrix();     // Whenever we get back here, we "pop" in order to restore the previous matrix state

// Repeat the same thing, only branch off to the "left" this time!

pushMatrix();

rotate(-theta);

line(0,0,0,-h);

translate(0,-h);

branch(h);

popMatrix();

}

}

Now use "Open..." to open the example in the Arduino library (in Processing).
Processing Folder -> libraries -> arduino -> examples -> arduino_input -> arduino_input.pde

Here's the code:

import processing.serial.*;

import cc.arduino.*;

Arduino arduino;

color off = color(4, 79, 111);

color on = color(84, 145, 158);

void setup() {

size(470, 280);

arduino = new Arduino(this, Arduino.list()[0], 57600);

for (int i = 0; i <= 13; i++)

arduino.pinMode(i, Arduino.INPUT);

}

void draw() {

background(off);

stroke(on);

for (int i = 0; i <= 13; i++) {

if (arduino.digitalRead(i) == Arduino.HIGH)

fill(on);

else

fill(off);

rect(420 - i * 30, 30, 20, 20);

}

for (int i = 0; i <= 5; i++) {

ellipse(280 + i * 30, 240, arduino.analogRead(i) / 16, arduino.analogRead(i) / 16);

}

}

Without explaining the graphics in this sketch (I'll leave that to you, dear reader) Let's see what is necessary to get Processing talking to the Arduino via Firmata.

First there are some import lines to reference the arduino and serial libraries, and a data type definition, these three lines are necessary somewhere at the top of the sketch.

import processing.serial.*; // reference the serial library

import cc.arduino.*; // reference the arduino library

Arduino arduino; // create a variable arduino of the Arduino data type

Then in the setup function this line is required

arduino = new Arduino(this, Arduino.list()[0], 57600);

Let's paste them into the tree sketch. Look over the tree sketch below with the new lines in them. [Sorry this wiki formatting is going to be ugly].

Finally we add an Arduino library function, arduino.analogRead(0), to replace the input from the mouse.

Just hook up a potentiometer or some other variable voltage input to analog pin 0 on your Freeduino and you've got the sketch moving with Arduino power.

Here's the code, with red highlighting for lines that have been changed or added.

/*

* Recursive Tree

* by Daniel Shiffman

*

* Renders a simple tree-like structure via recursion

* Branching angle calculated as a function of horizontal mouse  location

*/

import processing.serial.*; // reference the serial library

import cc.arduino.*; // reference the arduino library

Arduino arduino; // create a variable arduino of the Arduino data type

float theta;

void setup() {

size(200,200);

smooth();

println(Serial.list()); // List all the available serial ports:

arduino = new Arduino(this, Arduino.list()[0], 57600);

}

void draw() {

background(0);

frameRate(30);

stroke(255);

// Let's pick an angle 0 to 90 degrees based on the mouse position

/* float a = (mouseX / (float) width) * 90f; // original line */

float a = (arduino.analogRead(0) / (float) width) * 90f;

// Convert it to radians

theta = radians(a);

// Start the tree from the bottom of the screen

translate(width/2,height);

// Draw a line 60 pixels

line(0,0,0,-60);

// Move to the end of that line

translate(0,-60);

// Start the recursive branching!

branch(60);

}

void branch(float h) {

// Each branch will be 2/3rds the size of the previous one

h *= 0.66f;

// All recursive functions must have an exit condition!!!!

// Here, ours is when the length of the branch is 2 pixels or less

if (h > 2) {

pushMatrix();    // Save the current state of transformation (i.e. where are we now)

rotate(theta);   // Rotate by theta

line(0,0,0,-h);  // Draw the branch

translate(0,-h); // Move to the end of the branch

branch(h);       // Ok, now call myself to draw two new branches!!

popMatrix();     // Whenever we get back here, we "pop" in order to restore the previous matrix state

// Repeat the same thing, only branch off to the "left" this time!

pushMatrix();

rotate(-theta);

line(0,0,0,-h);

translate(0,-h);

branch(h);

popMatrix();

}

}

Note that the math is not the same as the original mouse action, but I'll also leave this to the reader to work out - some very interesting things happen with the tree anyway.

Now try out the other Firmata examples and functions and you'll be up and running with Firmata in no time.

## Arduino/Processing Comparación de lenguajes

El lenguaje de Arduino (basado en Wiring) está implementado en C/C++, y por tanto tiene algunas diferencias con Processing, el cual está basado en Java.

### Matrices (Array)

| Arduino | Processing |
| --- | --- |
| int bar[8]; bar[0] = 1; | int[] bar = new int[8]; bar[0] = 1; |
| int foo[] = { 0, 1, 2 }; | int foo[] = { 0, 1, 2 }; o int[] foo = { 0, 1, 2 }; |

### Bucles

| Arduino | Processing |
| --- | --- |
| int i; for (i = 0; i < 5; i++) { ... } | for (int i = 0; i < 5; i++) { ... } |

### Impresión

| Arduino | Processing |
| --- | --- |
| Serial.println("hola mundo"); | println("hola mundo"); |
| int i = 5; Serial.println(i); | int i = 5; println(i); |
| int i = 5; Serial.print("i = "); Serial.print(i); Serial.println(); | int i = 5; println("i = " + i); |

Página principal Referencia

Correcciones, sugerencias, y nueva documentación deberán ser publicadas en el Foro (castellano) o en el Foro (inglés).

El texto de la referencia de Arduino está publicado bajo la licencia Creative Commons Reconocimiento-Compartir bajo la misma licencia 3.0. Los ejemplos de código de la referencia están liberados al dominio público.